| Title | Make and repair a boat cockpit canopy and side curtains |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 10 |

| Purpose | People credited with this unit standard are able to: prepare to make; make; prepare to repair; and repair, a boat cockpit canopy and side curtains. |
| --- | --- |

| Classification | Industrial Textile Fabrication > Industrial Textile Fabrication Core Skills |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

Guidance information

- Evidence presented for assessment against this unit standard must be consistent with safe working practices and be in accordance with applicable manufacturer’s information, company requirements, industry guidelines and legislative requirements.  This includes the knowledge and use of suitable tools and equipment.

- Legislation, regulations and/or industry standards relevant to this unit standard include but are not limited to the Health and Safety at Work Act 2015; and any subsequent amendments and replacements.

3	Definitions

Company requirements refers to instructions to staff on policy and procedures that are available in the workplace.  These requirements may include – company policies and procedures, work instructions, product quality specifications and legislative requirements.

Industry guidelines refers to those practices and procedures commonly used as standard procedures to produce items of acceptable merchantable quality in the industrial textile fabrication industry such as related textbook descriptors.

Manufacturer’s information refers to technical information for a machine or product detailing operation; installation and servicing procedures; manufacturer instructions; technical terms and descriptions; and detailed illustrations.

Outcomes and performance criteria

Outcome 1

Prepare to make a boat cockpit canopy and side curtains.

Performance criteria

1.1	Materials that make up the canopy and curtains are identified.

Range	may include – fabric, frame, windows, bows, webbings or pads, hood, side linings, fastenings.

1.2	Customer requirements and job specifications are identified and verified.

1.3	A template is made.

Outcome 2

Make a boat cockpit canopy and side curtains.

Performance criteria

2.1	Fabric is identified and marked out.

2.2	Material is cut to size within specified tolerances.

2.3	Sections are attached.

Range	straight and even, curves and bends uniform, no loose or missed stitches, seams overlap and shed water, weather proof.

2.4	Windows are identified, aligned, and fitted.

2.5	Frame material is identified, assembled, and fitted to the boat.

2.6	Fabric is attached to frame.

Range	may include – secure attachment, no loose threads, no loose stitching, no wrinkles, no creases, no damage, lining clean, weatherproof.

2.7	Fabric is cleaned.

Outcome 3

Prepare to repair a boat cockpit canopy and side curtains.

Performance criteria

3.1	Damage and repair procedures are identified.

Range	may include – fabric identification, side curtains, webbings, padding, framing, windows.

3.2	Canopy is removed from the frame.

Outcome 4

Repair a boat cockpit canopy and side curtains.

Performance criteria

4.1	Replacement fabric to repair damage is identified.

4.2	Fabric is cut, assembled, stitched, and/or bonded.

Range	sewing machine, hand sewing, bonding;

stitching even, joints flat and even.

4.3	Damaged section is repaired.

Range	may include – straight and even, curves and bends uniform, no loose or missed stitches.

4.4	Damaged windows are identified and replaced.

4.5	Frame damage is identified and repaired.

4.6	Webbings and pads are repaired.

4.7	Fabric is attached to frame, and the frame operates.

Range	may include – secure; no loose threads, loose stitching, wrinkles, creases, damage; lining clean; weatherproof.

4.8	Fabric is cleaned.

| Planned review date | 31 December 2025 |
| --- | --- |

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 25 September 1997 | 31 December 2023 |
| Revision | 2 | 14 November 2000 | 31 December 2023 |
| Revision | 3 | 16 October 2003 | 31 December 2023 |
| Review | 4 | 26 March 2007 | 31 December 2023 |
| Rollover and Revision | 5 | 27 January 2015 | 31 December 2023 |
| Review | 6 |  | N/A |

| Consent and Moderation Requirements (CMR) reference | 0014 |
| --- | --- |

This CMR can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Comments on this unit standard

Please contact MITO New Zealand Incorporated info@mito.org.nz if you wish to suggest changes to the content of this unit standard.